ExoHand

Perfboard Assembly Guide

A Complete, Zero-Experience, Step-by-Step Build Manual

EMG-Controlled Robotic Exoskeleton Hand

# Table of Contents

# 0. Before You Start

This guide assumes you have never soldered a circuit before and have never built electronics from scratch. Every step is written out in full. Nothing is assumed. Read the entire guide once, start to finish, before you pick up a soldering iron. Building in the wrong order is the single biggest cause of failed boards.

This guide builds the ExoHand control board on perfboard (a board full of pre-drilled holes with copper pads, NOT a breadboard, and NOT a custom PCB). Perfboard requires you to manually wire every connection, either with solder bridges or short wire jumpers. This is slower than a breadboard but far more reliable for a competition demo, since nothing falls out.

Why This Order Matters Power wiring is built first, and tested alone, before any chip touches the board. Each section ends with a test. Do not skip a test to save time. A mistake found at Step 2 takes 2 minutes to fix. The same mistake found at Step 9 can mean rebuilding the whole board.

## 0.1 What You Are Building

The ExoHand control board reads a muscle signal from your forearm (EMG), decides how much you are flexing, and moves four servo motors that pull tendons in a robotic hand. The board has five sections that all connect together:

- Power section — battery, buck converter, two voltage rails (5V logic, 6V servo power), and one common ground.

- Brain section — Teensy 4.0 microcontroller, reads the EMG sensor and decides servo angles.

- Servo driver section — PCA9685 module, takes commands from the Teensy and generates the actual PWM signals that drive servos.

- Sensor section — one OYMotion analog EMG sensor (DFRobot SEN0240), reads muscle activity.

- Wireless section — WEMOS C3 Mini, broadcasts a WiFi page so you can watch the EMG signal live on a phone or laptop.

# 1. Safety First

- Never solder with the LiPo battery connected. Always build and test with a bench power supply first.

- LiPo batteries can catch fire if shorted. Keep the battery disconnected until the entire board is verified.

- Work in a ventilated area. Solder fumes should not be inhaled directly.

- Let the soldering iron tip touch only solder joints, never wires you are holding, never the battery.

- Unplug the soldering iron when not actively soldering, even for a 5-minute break.

- Servos can pinch fingers. Keep hands clear of the mechanical hand while testing motion.

# 2. Complete Parts List

Confirm you physically have every item below before starting. Running out of wire halfway through a power rail is a common beginner mistake.

| Qty | Component | Notes |
| --- | --- | --- |
| 1 | Perfboard | At least 7cm x 9cm, with copper pads on one side. |
| 1 | Teensy 4.0 | Main microcontroller. |
| 1 | WEMOS C3 Mini (ESP32-C3) | Wireless module for live EMG graph. |
| 1 | PCA9685 servo driver (HW-176 module) | Pre-built breakout board, not a bare chip. |
| 4 | MG996R servo motors | One per finger tendon. |
| 1 | OYMotion analog EMG sensor (DFRobot SEN0240) | Single channel, drives all 4 servos together. |
| 1 | 2S LiPo battery | 7.4V nominal. |
| 1 | Buck converter (adjustable output) | Steps battery voltage down to usable rail voltage. |
| 1 roll | 22-24 AWG hookup wire, multiple colors | Red for power, black for ground, other colors for signals. |
| 1 | Soldering iron + stand | Fine tip, 15-30W. |
| 1 roll | 60/40 rosin-core solder | 0.6-0.8mm diameter is easiest for beginners. |
| 1 | Wire strippers | 22-24 AWG range. |
| 1 | Multimeter | Must have continuity and DC voltage modes. |
| 1 | Bench power supply (adjustable, current-limited) | Used for safe first power-on, before the LiPo. |
| 1 | Small flathead screwdriver | For terminal blocks if used. |
| 1 | Heat shrink tubing or electrical tape | For insulating exposed joints. |
| 1 | Helping-hands clamp or tape | To hold wires still while soldering. |

# 3. Workspace Setup

- Work on a flat, non-flammable surface. A piece of scrap wood or a silicone mat is ideal.

- Set the soldering iron to 320-350°C if it has a temperature dial. If not, allow 2-3 minutes to fully heat before use.

- Lay out all components next to the perfboard, in roughly the position they will be soldered, before touching the iron. This is your map.

- Set the multimeter to continuity mode (the symbol that looks like a sound wave) and confirm it beeps when you touch the two probes together. This confirms the meter works before you rely on it.

# 4. Perfboard Basics (Read This Even If You Skip Other Sections)

Perfboard is a grid of holes. Each hole has a small round copper pad on the underside (the side you solder from). Pads are NOT connected to each other automatically, unlike a breadboard. Every single connection between two holes must be made by you, either with a blob of solder bridging two adjacent pads, or with a short wire soldered between two pads.

### 4.1 Two Ways to Make a Connection

- Solder bridge: for two holes that are directly next to each other, you can melt a connected blob of solder between their pads. Only use this for very short, low-current connections like signal wires.

- Jumper wire: strip a short length of hookup wire, bend the bare ends into small hooks, insert through both holes from the top, then solder each end from the bottom. Use this for every power, ground, and longer signal connection.

### 4.2 Component Orientation

Insert all chips, headers, and connectors from the TOP (component side) of the board. All soldering happens from the BOTTOM (copper side). Components should sit flush against the top surface before you solder, or they will wobble and crack joints later.

### 4.3 A Good Solder Joint

A good joint looks like a small shiny volcano shape, not a ball, not a blob, and not dull/grey. Touch the iron to BOTH the pad and the wire/lead at the same time for 2-3 seconds, then feed solder into the joint (not onto the iron tip), then remove the iron. Let it cool untouched for 5 seconds.

Common Beginner Mistakes Cold joint: dull, grey, crumbly looking solder. Usually means the iron was not hot enough or held in place long enough. Reheat and redo. Solder bridge you didn't mean to make: two adjacent pads accidentally joined. Check under magnification or with continuity testing. Component soldered in backwards. Check polarity markings (silkscreen, plus signs, color-coded wires) before soldering anything irreversible.

# 5. Planning the Board Layout

Before any soldering, decide where each section of the circuit will physically sit on the board. This prevents wires crossing over each other excessively and makes later debugging far easier.

| Zone | Location on board | Contains |
| --- | --- | --- |
| Power zone | One long edge of the board | Battery connector, buck converter, 5V rail strip, 6V rail strip, ground strip |
| Logic zone | Center of the board | Teensy 4.0, PCA9685 module |
| Wireless zone | Corner nearest an edge (for antenna clearance) | WEMOS C3 Mini |
| Sensor zone | Edge nearest where the EMG cable will exit the board | 3-pin header for the EMG sensor cable |

Draw this layout on paper first, full size, matching your actual perfboard dimensions. Mark where each component sits. This 10-minute step prevents the most common perfboard mistake: running out of room halfway through.

# 6. Step 1 — Build the Power Rails

, with no other chip connected. A power mistake found now is harmless. The same mistake found after the Teensy is soldered in can destroy the Teensy.

## 6.1 What You Are Building

Two separate power strips made of jumper wire, running the length of the power zone:

- GND strip — one continuous run of wire, soldered to every ground point on the board. This is your single common ground.

- 5V LOGIC strip — carries 5V to the Teensy, the PCA9685 logic pin, the WEMOS, and the EMG sensor.

- 6V SERVO strip — carries 6V to the PCA9685 servo power pin only. Servos draw this through the PCA9685's V+ rail.

## 6.2 Wiring Steps

- Solder the battery connector's two wires to the perfboard at the start of the power zone. Positive wire to one pad, negative wire to an adjacent pad. Mark these with a small dot of red (positive) and black (negative) marker or nail polish if available.

- Solder the buck converter's input pins (IN+ and IN-) to those same two pads. IN+ to the battery positive pad, IN- to the battery negative pad.

- Set the buck converter output BEFORE wiring anything else to it. Connect it temporarily to your bench power supply at a safe low voltage (or briefly to the battery if you must), and turn the onboard adjustment screw while measuring output with the multimeter, until it reads exactly 6.0V. Disconnect again before continuing.

- Solder a jumper wire from the buck converter OUT+ pad to the start of your 6V SERVO strip.

- Solder a jumper wire from the buck converter OUT- pad to the start of your GND strip.

- You need a SEPARATE 5V LOGIC rail. If your buck converter is single-output, add a second small buck converter (or a 5V linear regulator) fed from the same 6V SERVO strip, outputting 5V into a new 5V LOGIC strip. Do not run logic directly off 6V — it can damage the Teensy and WEMOS.

- Extend the GND strip down the full length of the power zone with jumper wire, so every future component has a nearby ground point to solder to.

## 6.3 Test Before Continuing

Power-Only Test (mandatory) Connect ONLY the battery (or bench supply set to 7.4V, current limit at 1A). Multimeter on the 6V SERVO strip: must read 5.8-6.2V. Multimeter on the 5V LOGIC strip: must read 4.8-5.2V. Multimeter between any GND point and any other GND point: must read 0V (continuity beep). If voltage sags or the supply hits its current limit, STOP. Recheck for a solder bridge between a power pad and a ground pad before continuing.

# 7. Step 2 — Wire the PCA9685 Servo Driver

The PCA9685 (HW-176 breakout) takes simple commands from the Teensy over I2C and generates 16 independent PWM signals. You will use 4 of its 16 channels.

## 7.1 Pinout Reference

| Pin | Function | Connects To |
| --- | --- | --- |
| VCC | Logic power for the chip itself | 5V LOGIC strip |
| GND | Ground | GND strip |
| V+ | Power for servo motors, passed through to channel outputs | 6V SERVO strip |
| OE | Output Enable, active LOW | GND strip (tie permanently low) |
| SDA | I2C data line | Teensy pin 18 |
| SCL | I2C clock line | Teensy pin 19 |
| CH0-CH3 signal pins | PWM output to servos | Servo signal wires (one channel per servo) |
| CH0-CH3 V+/GND pins | Power feed-through to servos | Servo power and ground wires |

## 7.2 Wiring Steps

- Place the PCA9685 module flat in the logic zone, header pins facing down toward the perfboard holes.

- Solder VCC to the 5V LOGIC strip.

- Solder GND to the GND strip.

- Solder V+ to the 6V SERVO strip. Confirm with multimeter this is NOT the same strip as VCC.

- Solder OE directly to the GND strip with a short jumper. This permanently enables the outputs.

- Leave SDA, SCL, and the channel pins unconnected for now. They get wired in later steps.

## 7.3 Test Before Continuing

PCA9685 Power Test Power the board from the bench supply again. Multimeter on PCA9685 VCC pin: should read 4.8-5.2V. Multimeter on PCA9685 V+ pin: should read 5.8-6.2V. If a small LED on the module lights up, that is normal and expected.

# 8. Step 3 — Wire the Teensy 4.0

## 8.1 Pinout Reference

| Teensy Pin | Function | Connects To |
| --- | --- | --- |
| VIN | 5V power input | 5V LOGIC strip |
| GND | Ground (use any one GND pin) | GND strip |
| Pin 18 (SDA) | I2C data | PCA9685 SDA |
| Pin 19 (SCL) | I2C clock | PCA9685 SCL |
| A0 | Analog input from EMG sensor | EMG sensor signal wire |
| Pin 0 (RX1) | UART receive | WEMOS C3 TX pin |
| Pin 1 (TX1) | UART transmit | WEMOS C3 RX pin |

## 8.2 Wiring Steps

- Solder two short headers or wire leads, one to VIN, one to a GND pin, both running to the respective power strips.

- Solder a jumper from Teensy pin 18 to PCA9685 SDA.

- Solder a jumper from Teensy pin 19 to PCA9685 SCL.

- Add one 4.7k ohm pull-up resistor from SDA to the 5V LOGIC strip, and one 4.7k ohm pull-up resistor from SCL to the 5V LOGIC strip. Many PCA9685 breakouts already include these onboard; check your module's silkscreen or datasheet. If unsure, add them anyway — extra pull-ups in this range do not cause harm.

- Leave A0, pin 0, and pin 1 unconnected for now.

## 8.3 Test Before Continuing

I2C Communication Test Power the board from the bench supply. Connect the Teensy to your computer with USB. Upload a basic I2C scanner sketch (search 'Arduino I2C scanner example' if you don't already have one). Open the Serial Monitor. You must see address 0x40 reported. That is the PCA9685. If nothing is found, double check SDA/SCL are not swapped, and that GND is truly common between Teensy and PCA9685.

# 9. Step 4 — Wire the WEMOS C3 Mini

| WEMOS Pin |  | Function | Connects To |
| --- | --- | --- | --- |
| VBus / 5V |  | Power input | 5V LOGIC strip |
| GND |  | Ground | GND strip |
| TX |  | Serial transmit | Teensy pin 0 (RX1) |
| RX |  | Serial receive | Teensy pin 1 (TX1) |
| Important: Cross the Wires TX always connects to RX, and RX always connects to TX, never TX-to-TX. If the Teensy and WEMOS cannot talk to each other later, this is the first thing to recheck. |  |  |  |

- Place the WEMOS in the wireless zone, away from servo wiring (servo motors can introduce electrical noise that affects WiFi).

- Solder VBus to the 5V LOGIC strip.

- Solder GND to the GND strip.

- Solder TX to Teensy pin 0 (RX1).

- Solder RX to Teensy pin 1 (TX1).

# 10. Step 5 — Wire the EMG Sensor

This board uses one OYMotion analog EMG sensor (DFRobot SEN0240). It outputs a raw analog signal centered at 1.5V, swinging between 0V and 3.0V depending on muscle activity. One sensor drives all four servos together — they all move to the same angle based on a single flex signal.

| Sensor Pin | Function | Connects To |
| --- | --- | --- |
| VCC | Power, 3.3-5.5V accepted | 5V LOGIC strip |
| GND | Ground | GND strip |
| SIG (signal/output) | Analog EMG output | Teensy pin A0 |

- Mount a 3-pin header in the sensor zone for the sensor's cable connector.

- Solder the VCC pin of the header to the 5V LOGIC strip.

- Solder the GND pin of the header to the GND strip.

- Solder the SIG pin of the header to Teensy A0 with a jumper wire.

- Do not let the SIG wire run parallel and touching alongside a servo power wire for any long stretch — this picks up electrical noise. Keep them separated where possible.

# 11. Step 6 — Wire the Four Servos

Each MG996R servo has three wires: signal (usually orange/yellow), power (usually red), and ground (usually brown/black). Confirm your specific servo's color code with its datasheet before soldering, since colors are not universal.

| Servo | PCA9685 Channel | Servo Wire | Connects To |
| --- | --- | --- | --- |
| Servo 1 | CH0 | Signal | PCA9685 CH0 signal pin |
| Servo 1 | CH0 | Power | 6V SERVO strip (via PCA9685 V+ row) |
| Servo 1 | CH0 | Ground | GND strip |
| Servo 2 | CH1 | Signal / Power / Ground | Same pattern, channel 1 |
| Servo 3 | CH2 | Signal / Power / Ground | Same pattern, channel 2 |
| Servo 4 | CH3 | Signal / Power / Ground | Same pattern, channel 3 |

- Mount four 3-pin headers in the servo zone, one per servo, spaced enough apart that connector plugs do not touch.

- For each header, solder the signal pin to the matching PCA9685 channel's signal pad.

- For each header, solder the power pin to the 6V SERVO strip.

- For each header, solder the ground pin to the GND strip.

- Double check channel order matches your firmware (CH0, CH1, CH2, CH3 = Servo 1, 2, 3, 4 in the code).

# 12. Step 7 — Full Pre-Power Checklist

Complete every line below with the board fully assembled, but with NOTHING powered on. Use only the multimeter.

| # |  | Check | Expected Result |
| --- | --- | --- | --- |
| 1 |  | Continuity: 5V LOGIC strip to GND strip | NO beep (open circuit) |
| 2 |  | Continuity: 6V SERVO strip to GND strip | NO beep |
| 3 |  | Continuity: 5V LOGIC strip to 6V SERVO strip | NO beep (these must stay separate) |
| 4 |  | Continuity: any two GND points on the board | Beep (connected) |
| 5 |  | Visual: every solder joint is shiny, not dull/cracked | Pass by eye, magnify if possible |
| 6 |  | Visual: no stray solder blobs bridging adjacent pads | Pass by eye |
| 7 |  | Visual: PCA9685 OE pin wire goes to GND, not anywhere else | Confirmed |
| 8 |  | Visual: all 4 servo signal wires go to different channels | Confirmed |
| 9 |  | Visual: TX/RX between Teensy and WEMOS are crossed, not parallel | Confirmed |
| Do Not Skip Check 1, 2, and 3 These three catch the most common and most damaging perfboard mistake: a hidden solder bridge shorting power to ground. If any of these beep when they should not, you have a short. Find it before applying any power. |  |  |  |

# 13. Step 8 — First Power-On (Bench Supply, Not Battery)

- Set the bench power supply to 7.4V output, current limit at 1.0A, OUTPUT OFF.

- Connect the supply leads to the battery connector pads on the board (where the LiPo will eventually plug in).

- Turn the output ON. Watch the voltage and current readout for 5 full seconds.

- If voltage holds steady near 7.4V and current draw is under 300mA with no servos commanded, you are clear.

- If voltage sags and settles low, turn off immediately. This means current draw is hitting your limit — recheck for a short using the Pre-Power Checklist before trying again.

- Once stable, measure the 5V LOGIC strip (expect ~5V) and 6V SERVO strip (expect ~6V) with the board live.

- Only after this passes, connect one servo at a time to its header and recheck current draw stays reasonable (a single idle MG996R draws very little, well under 100mA at rest).

# 14. Step 9 — Upload Firmware and Test

- Connect the Teensy to your computer via USB and upload the EMG-to-servo control sketch.

- With the bench supply still powering the board, sit still for the calibration delay at boot. This records your resting EMG baseline.

- Flex the muscle under the sensor. All four servos should move together, smoothly, with no jitter.

- Release the flex. Servos should return to rest position without bouncing.

- Open the Arduino Serial Plotter at 115200 baud to see the live EMG envelope value as a single line, confirming the signal rises on flex and falls on rest.

- Power the WEMOS C3 Mini, connect to its WiFi access point from a phone or laptop, and open its IP address in a browser to view the live graph.

- Only after every test above passes on bench power, disconnect the bench supply and connect the 2S LiPo for a final live test.

# 15. Troubleshooting Guide

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| Servos don't move at all | PCA9685 not found on I2C, or OE not grounded, or code sends zero PWM at boot | Run I2C scanner, confirm 0x40. Confirm OE wired to GND. Add a forced initial PWM write in setup(). |
| Servos jitter constantly | No common ground between PCA9685 and servo power, or noisy EMG signal with no smoothing | Confirm shared GND. Add/raise envelope smoothing and deadband in firmware. |
| Voltage sags and settles low on power-up | Short circuit, or current limit set too low, or stalled servo drawing high current | Disconnect servos, retest board alone. Raise current limit gradually. Inspect for solder bridges. |
| Teensy and WEMOS don't communicate | TX/RX wired straight across instead of crossed | Swap so Teensy TX goes to WEMOS RX, and Teensy RX goes to WEMOS TX. |
| EMG signal flat, never rises on flex | Sensor VCC/GND swapped, or signal wire on wrong analog pin, or electrode placement off-muscle | Recheck wiring against pinout table. Reposition electrodes along the muscle fiber direction. |
| WiFi page won't load | Not connected to the WEMOS access point, or wrong IP typed | Reconnect phone/laptop WiFi to the WEMOS SSID first, then browse to its printed IP. |
| Board works on bench supply, fails on battery | LiPo voltage sagging under load, or battery near-empty, or buck converter input range mismatch | Charge battery fully. Confirm buck converter accepts your LiPo's voltage range. Recheck connector polarity. |

# 16. Final Assembly and Strain Relief

- Secure every wire that leaves the board (battery, servos, EMG cable) with a small zip tie or tape loop anchored to the board, so a tug on the cable does not pull directly on a solder joint.

- Cover any exposed solder joints near moving parts with heat shrink tubing or electrical tape.

- Label each servo header (1-4) and the EMG cable with a small piece of tape, matching the hand's fingers, so reassembly after transport is fast.

- Before the Innovation Fair demo, repeat the full Pre-Power Checklist one more time. Vibration from transport can crack a marginal joint.

- Bring the bench power supply and multimeter to the demo if possible. If anything misbehaves on-site, you can isolate battery-vs-board issues in seconds.

End of guide. Keep this document with the board for any future repairs or modifications.