ExoHand

Perfboard Build Manual

Step by step. Brick by brick. No prior skill needed.

Read this top to bottom. Do not skip. Each module builds on last one. Box ticks mean you stop and check before next step.

# Module 0 – The Plan

You build hand controller on perfboard. No PCB. Perfboard is enough for demo.

Big idea: plug, do not glue chips. Sockets hold modules. Module dies, you swap it. No resolder.

Golden Rule Socket everything. Never solder Teensy, WEMOS, or PCA9685 straight to board. Use female headers. Modules drop in. Pull out if dead.

# Module 1 – Your Bricks

Lay these out first. Confirm each one in hand.

| Brick | What it is | How many |
| --- | --- | --- |
| Teensy 4.0 | Main brain board | 1 |
| WEMOS C3 Mini | Wi-Fi helper board | 1 |
| PCA9685 | Servo driver chip | 1 (not in schematic yet) |
| EMG channels | Muscle signal inputs | 3 |
| Servo connectors | Motor plugs | 4 |
| Buck / battery nets | Power supply lines | as drawn |
| I2C bus | Two-wire chat line | 1 |

Missing brick PCA9685 not in schematic yet. Add it. It drives 4 servos over I2C. Without it, servos do not move.

# Module 2 – Your Tools

- Soldering iron. Hot tip melts metal glue.

- Solder wire. The metal glue.

- Perfboard. Grid board with holes.

- Female headers. Plastic strips with sockets.

- DIP adapter. Small carrier board for PCA9685.

- Screw terminals or 3-pin headers. For servo plugs.

- Solid copper wire. For power rails.

- Hot glue gun. For holding wires down.

# Module 3 – Build It

## Step group A – Power rails

- Pick three rows along board edge.

- Run one solid copper wire down each. These are buses.

- Name them: VBAT rail, 5V rail, GND rail.

- Solder wire flat to copper. Long lines, no gaps.

Why rails Bus = shared power line. Every module taps same rail. Clean rails stop random resets.

## Step group B – Sockets

- Place female headers where each module sits.

- Solder headers only. Not the modules.

- Solder PCA9685 onto DIP adapter once.

- Plug DIP adapter into its socket on board.

Now Teensy, WEMOS, and PCA9685 all drop into sockets. None soldered direct.

## Step group C – Servos

- Add screw terminals or 3-pin headers for 4 servos.

- No loose jumper wires for servos. They pull free.

- Each servo: signal, 5V, GND. Match to PCA9685 outputs.

## Step group D – Signals and I2C

- Wire 3 EMG channels to Teensy inputs.

- Wire I2C: two lines, SDA and SCL.

- Connect Teensy and PCA9685 on same I2C bus.

## Step group E – Lock it down

- Hot glue every wire that leaves the board.

- Glue gives strain relief. Tug does not break joint.

- Check no bare wires touch. Recheck rails.

Strain relief A pulled wire kills a demo. Glue blob holds wire to board. Cheap insurance. Do it every time.

# Module 4 – 5-Day Plan

Five days. Doable. Stick to the split.

| Day | Job | Done means |
| --- | --- | --- |
| 1 | Solder sockets. Lay power rails. | Headers fixed. Rails run. |
| 2 | Wire signals. Glue wires down. | All links wired. |
| 3 | Plug modules. Flash firmware. | Boards boot. I2C talks. |
| 4 | Test EMG. Test servos. | Gesture moves hand. |
| 5 | Stress test. Fix flaky bits. | Survives 20 runs. |

# Module 5 – Faster But Risky

Breadboard option: zero soldering. Push pins into holes.

- Good: fast, no iron, easy swap.

- Bad: wires pull loose. Risky live on stage.

Verdict: perfboard wins for the fair. Use breadboard only to test early.

# Module 6 – Answer Before You Start

Tell me these. Then I tune the plan.

- Got perfboard and headers in hand already?

- Soldering iron available right now?

- Demo plan: live EMG gesture, or preset motion fallback?

# Module 7 – Word Bank

Plain words for every term above.

Perfboard

Flat board full of holes. You build on it.

PCB

Custom printed board. Made in a factory. You skip this.

Female header

Plastic strip of sockets. Modules plug in.

DIP adapter

Tiny board. Holds a chip so it fits a socket.

Bus / rail

Shared wire line. Carries power or data to all.

I2C

Two-wire chat line. Chips talk over SDA and SCL.

EMG

Muscle signal. Your arm flex becomes a number.

Servo

Small motor. Moves a finger to an angle.

Strain relief

Glue or clamp. Stops a tug from breaking a joint.

Firmware

Code on the board. Tells chips what to do.